N13

Sanitary appliances and fittings

Clauses - Not Used

Products

310 Wash basins generally

1.	Basin

1.1.	Manufacturer:  Multicare Medical Ltd

1.1.1.	Contact details

1.1.1.1.	Address:  Progress House
	Jacknell Road
	Dodwells Bridge Industrial Estate
	Hinckley
	Leicestershire
	LE10 3BS

1.1.1.2.	Telephone:  0800 298 6000

1.1.1.3.	Web:  multicaremedical.co.uk

1.1.1.4.	Email:  sales@multicaremedical.co.uk

1.1.2.	Product reference:  Matrix Height Adjustable Basin

1.2.	Form:  Wall mounted, wheelchair friendly, height adjustable wash basin.

1.3.	Operation:  Electric height adjustment by inset front push-button controls.

1.4.	Height adjustment range:  670 - 970 mm.

1.5.	Width:  780 - 1200 mm, subject to selected basin size.

1.6.	Width variations:  900 mm or 1200 mm.

1.7.	Basin options:

1.7.1.	Square or curved front basin.

1.7.2.	Medium curved front basin with integrated handrails.

1.7.3.	Overflow:  Available where specified.

1.8.	Taps:

1.8.1.	TMV3 long lever taps with built-in thermostatic protection; or

1.8.2.	Infra-red, gesture-operated, thermostatically controlled taps.

1.9.	Safety:  Safety stop to reduce crushing risk during adjustment.

1.10.	Finish:  White protective cover to conceal plumbing and electrics.

1.11.	Materials:  Aluminium basin lifter and high quality damage resistant materials.

1.12.	Services:  Installation, commissioning, maintenance, servicing and training available from Multicare.

1.13.	Warranty:  1 year.

Specifier note: Confirm final basin size, front profile, tap option, overflow requirement, wall structure, services and setting-out with Multicare prior to installation.